Chapter 5: Disassembly

# Disassembling the Base Unit

These are the directions for disassembling the base unit. You will need a 6mm Nut Driver, a medium size "+" Screw driver, a chip tool and wire cutters.

These directions are to disassemble the complete unit and will be cross-referenced to Section 7 for the replacement of component parts.

## Remove Battery Module

- Slide the latch lock in direction of the arrow. Battery module will pop-up automatically.

- Now, gently pull the module out of its housing.

## Remove ODD (CD-ROM/DVD-ROM/CD-RW…) Module

- Slide the latch lock in direction of the arrow. Gently pull the module out of its housing.

## Remove HDD Module

- Turn the power off and turn the notebook upside down with the back facing you. Remove 1 screws (M2.5X 3).

- To draw out on the HDD module.

- Remove 4 screws (M3.0X4) on the HDD carrier.

- Remove the HDD shield.

## Remove DDR RAM

- Remove 2 screws (M2.5X3) holding Ram door in place.Gently pick Ram door up.

- Unlock the DDR  SODIMM then pick it up.

## Remove Keyboard

- Turn the power off and remove the strip cover.

- Remove 2 screws (M2.5X3).

- Disconnect the keyboard FPC connector from mother board.

## Remove LCD Module

- Lift strip cover up from the right side, and remove it.

- Remove 1 screw (M2.5X3) on co-axial cable.

- Disconnect the co-axial cable.

- Remove 4 screws(M2.5X6).

- Disassembly the LCD module from logic base.

## Disassemble the Display and Inverter Board

For a start Remove LCD Module

- Remove 2 screws pads and 2 rubber pads.

- Remove 15”LCD-4 screws(M2.5X6);14”LCD-2 screws(M2.5X6)holding the LCD bezel in place.

- Remove LCD Bezel.

- Remove 1 screws (M2.5X3) mounting the inverter board.

- Remove 2 screws (M2.5X6) mounting the LCD panel subassembly on LCD cover.

- Disassembly LCD panel subassembly from LCD cover.

- Disconnect the coxial cable connector from the LCD.

- Disconnect the LCD high voltage cable and FPC from the inverter board.

- Remove 4 or 8 screws (M2.0X3.0FP) mounting the LCD panel on LCD hinge brackets.

## Remove Thermal Module and CPU

For a start Remove Keyboard

- Remove 1 screw (M2.5X15), 1 screw (M2.5X3), power BTN and disconnect power PCB.

- Remove 1 screw (M2.5X6) on the Thermal plate, and remove Thermal plate.

- Use the opposite direction release 4 screws. (M2.5X6), and disconnect the Thermal module cable.

- Disassembly the Thermal module.

- Use “-“ screw driver disassembly the CPU.

## Remove Logic Upper

For a start to ,remove KB, remove LCD module, remove battery module, remove HDD module,remove ODD module, and remove Thermal Module

- Remove screws.

- Remove 5 screws (A:M2.5X11, B:M2.5X3, C:M2.5X6)

- Disassembly wireless cable antenna, Touch Pad FPC, speaker cable.

- Logic upper to separate.

## Remove Touch Pad board

For a start to remove LCD module

- Remove Logic upper.

- To tear open the adhesive tape.

- Remove 4 screws (A:M2.5X4, B:M2.5X3)

- Separate upper support.

- Disconnection the FPC.

## Remove Speaker

For a start to remove Logic Upper, remove Touch pad.

- Remove 2 screws (M2.5X3).

- Turn over and remove 4 screws (M2.5X6), 1 screw (M2.5X3) and 1 gasket.

- Disassembly saddle L & R.

- Remove 5 screws (M2.5X3).

- Remove up shield and disassembly speaker.

## Remove Mother Board

For a start to remove logic upper

- Remove 1 screws (M2.5X4) holding IO shield.

- Disconnect the LED connect cable.

- Disassemble mother board from main frame.