Chapter 4
DC-DC Converter

## Power

-  cells Li-Ion 18650 size smart battery Pack with 53.28Wh capacity

-  cells Li-Ion 18650 size smart battery Pack with 79.92Wh capacity

## DC-DC CONVERTER

HEL81 Series Power System block diagram

### Description

This specification defines the performance and characteristic of 90W AC adapter power supply. It supplies a constant voltage 19V output source for HEL81 series notebook computer.

### Feature

- Accepts universal input from 90V AC to 264V AC

- Offers constant Voltage 19V output source with 75W max output power capacity.

- High efficiency 83% min at 100Vac

- Compact Size

### Adapter Electrical Specification

OUTPUT Voltage Dynamic regulation: Output voltage within18.5-19.9V, load current 10%100%, frequency 100Hz, 50% duty cycle, recover time  1msec.

Input Voltage range:	universal input, 90VAC TO 264VAC

Inrush current:	150A Peak and no damage @220Vac

Input frequency range:	47~63Hz

Input Current:	2.0Amax at 100 VAC

Start-up time:	 3sec Max. @115Vac

HOLD-UP time:	10ms min. @115VAC,  full load condition

OVP: 	29V max. automatic shut down

Short circuit protection: Output can be shorted without damage, and auto recovery

OUTPUT Voltage Regulation: 18.5-20V including the effects of line Voltage variation, load current, ripple and noise

OUTPUT Current: Current: 0Amin, 4.74Amax continuous

OUTPUT Voltage ripple: 380mv PK-PK for full  load.

#### DC OUTPUT PIN OUT:

PIN1	Center Pin Adapter +output

PIN2	Barrel (Ring) Adapter returns.

Operating temperature:	0 °C TO 40 °C

Temperature Range:

Storage temperature:	-20 °C TO 65 °C

CAUTION

Danger of explosion if battery is incorrectly replaced.

Replace only with the same or equivalent type recommended by the manufacturer.

Dispose of used batteries according to the manufacturer’s instructi

### Description

The DC-DC converter is designed to supply the power for HEL81 series notebook computer of Compal. It supply +5VALWP, +3VALWP, +1.5VSP, +1.8VP, +2.5VSP, +VSBP, +0.9VSP, +1.2VSP, +VGA_COREP, for logical system, + CPU_CORE for CPU and supplies for the built-in KB910QF microprocessor which handles the keyboard and PMU control functions of the system. The power ON/OFF is controlled by KB910QF. There is also a built-in charger power source. It can charge battery pack whether the computer is ON or OFF.

### Features

#### High efficiency, up to 85% (using battery)

#### Accept wide range DC input voltage from 8V to 19V

#### Built-in charger power source

#### The power ON/OFF is controlled by software

### Electrical specification

#### Input Voltage/Current

##### 8V to19V at the summing point of AC-DC and battery

##### INPUT Current 10.7A max from battery

##### 4. 3A max from 90W AC-DC Adapter.

### Temperature Range:

#### Operating temperature : 0° C to 40° C

#### storage temperature range : -20° C to 65° CDC/DC OUTPUT

Item	+5VALWP	+CPU_CORE	+VGA_COREP

Fixed output voltage/Current

nominal voltage	+5V	depend on VID                         1.1V

min. current	0A	0A                                              0A

max. current	5A	36A	12A

peak current	6A	51.42A	17.14A

total regulation 	5V±5%	depend on VCC staticand        1.1V±5%

Transient Tolerance--

ripple voltage	100mVp-p max	10mVp-p max@36A          60mVp-p max

Item	+0.9VSP	+3VALWP	+1.05VSP

nominal voltage	+0.9V	+3.3V	+1.5V

min. current	0A	0A	0A

max. current	2A	5A	5A

peak current	6A	6A	6A

total regulation	0.9V±5%	3.3V±5%	1.5V±5%

ripple voltage	40mVp-p max	100mVp-p max	60mVp-p max.

Item	+2.5VSP	+VSBP                                  +1.8VP

nominal voltage	+2.5V	+1.05V                                    +1.8V

min. current	0A	0A                                           0A

max. current	350mA	0.12A                                       5A

peak current	3.2A                        0.2A                                        5.8A

total regulation	2.5V±5%                9V-20V                                 1.8V±5%

ripple voltage       100mVp-p max        300mVp-p max                     60mVp-p max

Item	+1.5VSP

nominal voltage	+1.5V

min. current	0A

max. current	5A

peak current	6A

total regulation	    1.5V±5%

ripple voltage	100mVp-p max                           .

VOLTAGE IDENTIFICATION CODES

| ### VID6 | ### VID5 | ### VID4 | ### VID3 | ### VID2 | ### VID1 | ### VID0 | ### VDAC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1.5000 |
| ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1.4875 |
| ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1.4750 |
| ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1.4625 |
| ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1.4500 |
| ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1.4375 |
| ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1.4250 |
| ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1.4125 |
| ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1.4000 |
| ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1.3875 |
| ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1.3750 |
| ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1.3625 |
| ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1.3500 |
| ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1.3375 |
| ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1.3250 |
| ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1.3125 |
| ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1.3000 |
| ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1.2875 |
| ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1.2750 |
| ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1.2625 |
| ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1.2500 |
| ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1.2375 |
| ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1.2250 |
| ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1.2125 |
| ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1.2000 |
| ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1.1875 |
| ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1.1750 |
| ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1.1625 |
| ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1.1500 |
| ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1.1375 |
| ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1.1250 |
| ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1.1125 |
| ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1.1000 |
| ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1.0875 |
|  | ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1.164V |
| ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1.0750 |
| ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1.0625 |
| ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1.0500 |
| ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1.0375 |
| ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1.0250 |
| ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1.0125 |
| ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1.0000 |
| ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0.9875 |
| ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0.9750 |
| ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0.9625 |
| ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0.9500 |
| ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0.9375 |
| ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0.9250 |
| ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0.9125 |
| ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0.9000 |
| ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0.8875 |
| ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0.8750 |
| ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0.8625 |
| ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0.8500 |
| ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0.8375 |
| ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0.8250 |
| ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0.8125 |
| ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0.8000 |
| ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0.7875 |
| ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0.7750 |
| ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0.7625 |
| ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0.7500 |
| ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0.7375 |
| ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0.7250 |
| ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0.7125 |
| ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0.7000 |
| ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0.6875 |
| ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0.6750 |
| ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0.6650 |
| ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0.6500 |
| ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0.6375 |
| ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0.6250 |
| ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0.6125 |
| ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0.6000 |
| ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0.5875 |
| ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1 |  | ### 0.5750 |
| ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0.5625 |
| ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0.5500 |
| ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0.5375 |
| ### 1 ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0.5250 |
| ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0.5125 |
| ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0.5000 |
| ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0.4875 |
| ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0.4750 |
| ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0.4625 |
| ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0.4500 |
| ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0.4375 |
| ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0.4250 |
| ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0.4125 |
| ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0.4000 |
| ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0.3875 |
| ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0.3750 |
| ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0.3625 |
| ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0.3500 |
| ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0.3375 |
| ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0.3250 |
| ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0.3125 |
| ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0.3000 |
| ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0.2875 |
| ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0.2750 |
| ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0.2625 |
| ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0.2500 |
| ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0.2375 |
| ### 1 ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0.2250 |
| ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0.2125 |
| ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0.2000 |
| ### 1 ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0.1875 |
| ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0.1750 |
| ### 1 ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0.1625 |
| ### 1 ### 11 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0.1500 |
| ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0.1375 |
| ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0.1250 |
| ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0.1125 |
| ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0 | ### 0.1000 |
| ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 1 | ### 0.8750 |
| ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0 | ### 0.7500 |
| ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 1 | ### 0.6250 |
| ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0 | ### 0.5000 |
| ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 1 | ### 0.3750 |
| ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0 | ### 0.2500 |
| ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 1 | ### 0.1250 |
| ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0 | ### 0 |
| ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 1 | ### 0 |
| ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0 | ### 0 |
| ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 1 | ### 0 |
| ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0 | ### 0 |
| ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 1 | ### 0 |
| ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0 | ### 0 |
| ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 1 | ### 0 |

### Charger

#### Controlled by KB910 microprocessor from motherboard

#### Temperature sense capability for the battery (charge active between 0°C~ 40°C)

#### Fast charge current 3Amps(max.) for Li-Ion Battery at  system off, approach 25W fast charge at  system ON. (depend on system load)

#### Trickle charge: Typical 500mA precharge current for Li-Ion Battery . All trickle charge are controlled by KB910.

#### Charge termination: When Fully-Charge bit is set,charger is terminated by KB910

#### When system is turned off , the charge time is 4.0 hrs typically  from empty to full for Li-Ion 9 cell battery  .When system is turned off, the charge time is 3.0 hrs typically  from empty to full for Li-Ion6 cell battery .

#### Other battery services are presented by KB910 microprocessor includes maximum charging timer, charging temperature range etc.

#### Charger power:

##### Constant current mode: 3.0A±8%

##### Constant adapter current mode: 4.3A±6% (For 90W system)

BATT+ Constant Voltage mode:    12.6V±1% for Li-Ion Battery.

### OVER Current protection:

1. +3VALWP: >4.4A.

2. +5VALWP: >5.3A.

3. CPU_CORE: >30A.

4. +1.8VP: >8.85A.

5. +1.5VSP: >7.42A.

6. +1.05VSP: >7.36A.

### OVER Voltage protection:

+5VALWP:	5V +(4% ~ 10%)

+3VALWP:	3.3V +(4% ~ 10%)

+CPU_CORE	over 2V of programmed VID level

### Under voltage protection

+5VALWP:	5V *(60% ~ 80%)

+3VALWP:	3.3V *(60% ~ 80%)

+CPU_CORE	Under 30% of programmed VID level

### Short circuit protection:

Latch mode for +5VALWP, +3VALWP, +CPU_CORE

### I/O

#### DC-Jack

Pin 2,3: Center pin Adapter power +input

Pin 4,5: Barrel (Ring) Adapter power return

Battery Connector

Pin 1: BATT+

Pin 2: BATT+

Pin 3: ID

Pin 4: B/I

Pin 5: SMC

Pin 6: SMD

Pin 7: TS

Pin 8: GND

Pin 9: GND

#### Interface between Power with M/B

| - DC/DC |  |  |  |
| --- | --- | --- | --- |
| - Signals | - I/O | - Voltage Level | - Description |
| - SUSP# | - I | - 0~3.3V | - Low Active, system suspend control signal |
| - 51ON# | - I | - 0~floating | - Low Active, POWER ON control signal. |
| - FSTCHG | - I | - 0~3.3V | - High Active, ENE910 use this pin to control the fast charge of charge |
| - ACOFF | - I | - 0~3.3V | - High Active, turn off the Adaptor power for battery automatic learning cycle |
| - ACIN | - O | - 0~3.3V | - High Active, provide to ENE910 to mean the Adaptor power is present |
| - VGATE | - O | - 0~3.3V | - High Active, it will go high when +CPU_CORE is ready |
| - VR_ON | - I | - 0~3.3V | - High Active, turn on/off the +CPU_VIDP |
| - BATT_TEMP | - O | - 0~3.3V | - Analog signal, ENE910 using this voltage level to calulate battery’s temperature |
| - IREF | - I | - 0~3.3V | - Analog signal, ENE910 using this voltage for setting charge current |
| - VID [0..6] | - I | - 0~3.3V | - The +CPU_CORE voltage depends on those PINs VID[6..0] |
| - CHGRTC | - O | - 3.3V | - Charge RTC-battery power source |
| - EC_SMB_DA, - EC_SMB_CK | - I/O | - 0~5V | - Interface of Smbus, communicate between ENE910 and smart battery |
| - SYSON | - I | - 0~3.3V | - High Active, ENE910 use this pin to control the SYSON signal |
| - BATT_AOVP | - O | - 0~3.3V | - Analog signal, ENE910 using this voltage level for battery over voltage protection |

### BATTERY

#### Li-Ion smart

18650 size, 3S2P, 12.6V/4800mAH

18650 size, 3S3P, 12.6V/7200mAH

##### More than 300 charging/discharging cycles for Li-ion battery.

##### Modularized battery pack, easy to be replaced.

#### On board RTC battery: Maxell ML1220T13 3V/14mAH Lithium

- Sanyo ML1220T28 3V/15mAH Lithium

- CAUTION

- Danger of explosion if battery is incorrectly replaced.

Replace only with the same or equivalent type recommended by the manufacturer.

Dispose of used batteries according to the manufacturer’s instructions.

## INVERTER SPECIFICATION

## HEL81 15.4inch INVERTER SPECIFICATION

### . Description

This inverter is designed to light up the CCFL of LCD for HEL81 notebook. This inverter is designed to light up the CCFL of LCD for notebook. It should be supported HEL81 LCD panels. There are two control signals that come from system to control lamp brightness. One signal is named DAC_BRIG, which limits current to meet LCD lamp current specification. Another one is named PWM, which adjusts lamp brightness. This inverter brightness is adjusted by PWM burst mode. The PWM burst mode is that turning on and off the lamp at a rate of 150Hz. The effective brightness is a function of the duty cycle.

1. Features

Wide range 9V to 21V input voltage

Brightness adjustment by PWM burst mode.

Close loop controls lamp current.

2. Absolute maximum rating

2.1. Environment: Temperature: Operating temperature: 0℃ ~ 55℃

Storage temperature : -20℃ ~ 70℃

Humidity:  0 ~ 90% without condensation

MTBF:  MIN 50000 hours.(In Compal system)

3. Electrical characteristic

| No | Item | Symbol | Min. | Typ. | Max. | Unit | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Input voltage | INV_PWR | 9 | 14.8 | 21 | V | 7.5V(continuous) can work *Note 1 |
| 2 | Input current | Iin | -- | 0.33 | -- | A |  |
| 3 | Lamp current | IL | 3.0 | -- | 6.8 | mA | DAC=0V *Note 2 |
| 4 | Lamp current | IL | 2.7 | -- | 6.3 | mA | ##### DAC=1V |
| 4 | Frequency | F | 45 | 55 | 65 | KHz | *Note3 |
| 5 | Output power | Pout | -- | -- | 4.5 | W |  |
| 6 | Efficiency |  | 80% | -- | -- | -- |  |
| 7 | Starting voltage | Vs | 1550 | -- | -- | V | At 0’C |
| 8 | Starting time | Tvs | 1 | -- | 1.5 | Sec |  |
| 9 | Dispoff# |  | 2.8 | 3.3 | 3.6 | V | Backlight on/off signal |
|  |  |  | 0 | 0.5 | 0.8 | V | Low level |
| 10 | Limited lamp maximum current | DAC-BRIG | 0 |  | 3.3 | V | *Note 2 |
| 11 | PWM signal *note 4 | INV_PWM | 142 | 150 | 158 | Hz | PWM signal frequency |
|  |  |  | 3.0 | 3.3 | 3.6 | V | PWM signal amplitude |
|  |  |  | 30 | -- | 100 | % | Duty=TonPeriod |
| 12 | lamp current over-shoot | IZero−PK | -- | -- | 10 | % | Line transient( 10.8V to 21V/100us) and turn on transient |
| 13 | Current Waveform factor | IpIrms | 1.27 | 2 | 1.56 | Multiple | OR I−pIrms *10 |
| 14 | Unbalance Rate | Ip−I−pIrms | -10% | 0 | +10% | Multiple |  |
| 15 | Turn off current (Hight side) | IHL | -- | -- | 0 | A | PWM=30% |
| 15 | Turn off voltage (Low side) | Voff | -- | -- | 150Vp-p | V | PWM=30% |
| 16 | Voltage Rise time (Low side) | Trise | -- | -- | 300us | us | PWM=30% |
| 17 | Voltage fall time(Low side) | Tfall | -- | -- | 300us | us | PWM=30% |

Notes:

*1 The inverter can work in 7.5V input voltae(continuous),but 7.5V electronic characteristic

will not be care.(Note: the display must be normal and can not glitter or become dark)

*2. Limited lamp maximum current by DAC_BRIG signal:

When DAC_BRIG voltage is 0V and INV_PWM enables (100%), lamp has max. limited current.

When DAC_BRIG voltage is 3.3V and INV_PWM enables (100%), lamp has min. limited current.

When add 1V DAC, the 100% Lamp current will decrease 0.5mA.

DAC_BRIG signal comes from system chipset with internal resistance of 3KΩ.

*3. Inverter operating frequency should be within specification (45~65kHz) at max. and min. brightness load.

*4. INV_PWM enable implies INV_PWM signal is High level (On duty cycle is 100%). It is a square wave of 150Hz to adjust backlight brightness that is a function of PWM duty cycle. Backlight brightness is maximum value under INV_PWM at 100% and brightness is minimum under INV_PWM at 30%.

*5. The system interface signals belong to 3.3V.

*6. Please make sure open lamp output voltage should be within starting voltage specification.

*7. Inverter should pass human body safety test.

*8. Inverter should no smoking by any component open/ short test

*9. Transformer voltage stress should not be over 85% under any condition

( turn on overshoot transient and line transient).

*10. Audio noise should be less than 36dB at 10 cm distance.

4. Electrical specification

4..1. Electrical specification

| No | Symbol | Min. | Typ. | Max. | Unit | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Voper. | -- | 650 | -- | Vrms | Lamp operating voltage(650+/-50) |
|  | IL | 6.2 | 6.5 | 6.8 | mArms | DAC_BRIG: 0 V, PWM: 100% |
|  | IL | 3.0 | 3.3 | 3.6 | mArms | DAC_BRIG: 0 V, PWM:30% |
|  | IL | 5.7 | 6 | 6.3 | mArms | DAC_BRIG: 1 V, PWM: 100% |
|  | IL | 2.7 | 3 | 3.3 | mArms | DAC_BRIG: 1 V, PWM:30% |
|  | f | 45 | 55 | 65 | KHz |  |
|  |  | 80% | -- | -- | -- |  |

4.2 Thermal

All components on inverter board should follow below rules:

- Component using conditions (component stress) must be within component specification including voltage rating, current rating, temperature etc.

- Component temperature should follow below:

  - Δ T < 30℃ , at 25 , 35℃.

  - Component temperature should be less than 70℃ inside system at 35℃.

5 Connector description

5.1. Input Connector:

CN1:  ACES 87213-0700; JST SM07B-SRSS-TB

| Pin No. | Symbol | Description |
| --- | --- | --- |
| 1 | INV_PWR | Input voltage (9V-21V) |
| 2 | INV_PWR | Input voltage (9V-21V) |
| 3 | INV_PWM | Adjust brightness by burst mode(3.3 V 150Hz) |
| 4 | DISOFF # | Backlight on/off control, active HIGH(3.3V) |
| 5 | DAC_BRIG | Max. current limit |
| 6 | GND | Power system return |
| 7 | GND | Power system return |

5.2. Output Connector:

CN2: JST_SM02B_BHSS-1

| Pin No. | Symbol | Description |
| --- | --- | --- |
| 1 | HV | Connected to high voltage of LCD lamp |
| 2 | LV | Connected to low voltage of LCD lamp |

Note : Please mark “ CAUTION HIGH VOLTAGE” around CN2

6. Safety Protection

6.1 Open lamp protection:

When inverter is on open lamp status, any component on inverter should be O.K and inverter is no damaged, no fire and no arcing. If inverter can’t shunt down during open lamp happen, inverter must pass below conditions:

- Human body test.

- Open lamp burning: Inverter burns for 24 hours at open lamp status. No parts damage.

6.2 Human body safety test:

Short inverter output, transformer secondary output to GND by a 2KΩ resistor which connects one end to GND and another one to those outputs. They should meet output current limitation requirement as follow. Output current I is the current that flows through 2KΩ resistor.

- Output current  I ≦ 0.7mA , if frequency f ≦ 1KHz

- Output current  I ≦ 0.7mA * f (kHz) , if f ≧ 1KHz.

However, output current should be less than 70mA even frequency is more than 100KHz.

6.3 Abnormal test:

Any one component is short or open; inverter should be no fire, no arcing. And result must meet output current limitation requirement.